Appendix 25

INSTRUCTIONS FOR

RELEASE OF DECAYED (non-radioactive) SOLID WASTE TO

ORDINARY REFUSE
SCOPE - This procedure is to be followed each time a container of solid radioactive waste has decayed to background and is to be released for ordinary refuse disposal.
PURPOSE – To ensure that each occurrence of decayed radioactive waste that is released for ordinary refuse disposal is properly handled.
PROCEDURE – This applies to the decay-in-storage (DIS) of radioactive solid waste.  You must have held the container of solid radioactive waste, usually, for 10 half lives from the date it was declared full before release to ordinary refuse is considered.

“Solid Waste Disposal” Form

Line 1: 
Enter the date that the radioactive waste container was declared “full”.
Line 2:
Enter the total activity put into the container (NOT decayed).

Line 3:
Enter zero (0) if the calculated final activity (decayed), based on information from lines 1 and 2, is less that 0.05 uCi.  If the final activity is 0.05 uCi or greater, STOP, return container to storage until the activity decays to less than 0.05 uCi.

[Lines 4, 5, 6, and 7 are per ODH 3701:1-38-19 (E)]
Line 4:
Make a check mark, indicating that nothing was found above background, after checking the container (all sides) with an approved survey meter.  Document what background is for the meter used just below the check mark.  The Radiation Safety Office (RSO) MUST be contacted and an RSO person present to observe the first container surveyed when following this procedure for the first time.  The first container released for ordinary refuse only and none thereafter will be observed by the RSO unless the need to do so is indicated or requested by you.  If any reading above background is found, STOP, return container to storage until a reading of background or less can be observed.
Line 5:
Make a check mark indicating that you visually inspected the container outside and inside contents, if possible, and that you removed or TOTALLY obliterated any label, sign or wording that is a radioactive material symbol or reference.
Line 6:

Enter the date that you released the decayed solid waste to ordinary refuse.

Line 7:
Enter your initials, indicating that you were the person who completed all items in this procedure.

8/06
I:\Radiation Safety\Manuals\RadSafetyManual\RadManAppendix25Instructions.doc
